Intel(R) Quick SPI

(HIDSPI over Touch Host Controller)

Driver Release Notes

WW34’2023

Driver Version: 4.0.0.93

# Kumar, Sudhir/Komalan, Mukesh/ Desai, Sunny

Contents

1.	Introduction	5

1.1	Document Scope	5

2.	Intel(R) Quick SPI Driver Package	6

3.	Supported Configuration	7

4.	Feature implementation/Fixes	8

5.	Known Limitations	9

6.	HW Setup with HIDSPI PCB	9

7.	BIOS Menu Configuration	11

Revision history

| Driver Revision Number | Description | Revision Work Week |
| --- | --- | --- |
| 4.0.0.29 | Initial release for Quick SPI driver based on MSFT HidSpiCx | WW51, 2021 |
| 4.0.0.30 | - Q1’22 Major quarterly release of Quick SPI driver | WW12, 2022 |
| 4.0.0.31 | - Driver not released/submitted to OWR, as requirement for Win 11 SV2 OS was with EWDK: 22621 Nickel RTM build, which wasn’t available in OWR for driver build with Jenkins build env |  |
| 4.0.0.32 | - Q2’22 Major quarterly release of Quickspi driver (EWDK: 22621 Nickel RTM build) | WW26, 2022 |
| 4.0.0.55 | - Q3’22 Minor release of Intel(R) Quick SPI driver (for MSFT) | WW33, 2022 |
| 4.0.0.57 | - Q3’22 Major quarterly release of Intel(R) Quick SPI driver. | WW38, 2022 |
| 4.0.0.68 | - MSFT PreProd signing included in Jenkins environment of Intel(R) Quick SPI Driver for MTL | WW46,2022 |
| 4.0.0.70 | - Q4’22 Major quarterly release of Intel(R) Quick SPI driver. | WW51’2022 |
| 4.0.0.83 | - Q1’23 Major quarterly release of Intel(R) Quick SPI driver. | WW13’2023 |
| 4.0.0.89 | - Q2’23 Major quarterly release of Intel(R) Quick SPI driver for MTL PV. | WW26’2023 |
| 4.0.0.93 | - Q3’23 First release of Intel(R) Quick SPI driver for MTL Windows10 Supports. | WW34’2023 |

Acronyms

| Term | Description |
| --- | --- |
| Quick SPI | Intel developed Windows driver for HidSpi over Touch Host Controller |
| THC | Touch Host Controller within the chipset that supports SPI based host controller interface to touch devices |
| SPI | Serial Peripheral Interface. Synchronous serial communication bus used to attach peripherals to motherboard. In the ADP-P and MTL chipset, there are two instances of THC-SPI host controllers. |
| ACPI | Advanced Configuration and Power Interface – for this specification we are assuming ACPI 5.0 |
| HidSpi | Microsoft defined specification that defines the protocol, procedure, and features for input devices to communicate using HID protocol over SPI interface |
| HidSCx | Microsoft defined Serial Class Extension framework for HID over Serial Buses. The class extension is defined for clients which implement v1.0 of hidspi protocol |
| SoC | System on Chip |
| WDK | Windows Driver Kit |
| RPL | Raptor Lake Platform |
| ADL | Alder Lake Platform |
| MTL | Meteor Lake Platform |
| WoT | Wake on Touch |

# Introduction

This document specifies the driver release notes for Quick SPI driver, which is based on Intel’s Touch Host Controller (THC) subsystem.

## Document Scope

This document was written to guide on Quick SPI Driver releases for Windows OS Win10, Win11 (SV1,SV2) on CCG platforms starting & supporting from MTL. This document will focus on the Driver package content, HW setup (used for validation of driver), BIOS menu config, driver changes (feature implementations/fixes) and known limitations.

# Intel(R) Quick SPI Driver Package

Driver Version: 4.0.0.93

Content Overview

# Supported Configuration

Target Platforms:

- MTL-M/P/H/U

- MSFT Platforms

Windows OS:

- Win 11 SV2

- Win10 Supports

EWDK:

- 22621.NI.4 (RTM build)

- 19041.1 (RTM build)

HIDSPI Panel used for validation:

- Wacom (BOM6)

SPI Interface cable:

- 20-16 pin SPI (1.8V)

# Feature implementation/Fixes

- HSD:16021604585, Function event violating the IRQL level of residing function block, which was having higher IRQL level, which in turn causing the BSOD

- HSD:14019759256, [MTL] THC SW to set Active LTR to minimize impact SoC Low power residency.

- Enhanced support for WIN 10 19041 (20H1)

- Fix of WoT not enabled for WIN 10

# Known Limitations

- This driver package contains: Prod signed (Intel Signed, HLK and MSFT HDC signed) for Desktop_Prod driver build.

- Data corruption when Read & Write Max Packet Size set to 64 Bytes and Read & Write Streaming Mode Enabled.

- Wake On Touch supports only for Win11.

- WoT (Wake using Touchscreen) feature is disabled by default.

Steps to enable WoT:

- Ensure base INF is installed (QuickSPI-4.0.0.93\Desktop_Prod\x64\Release\IntelQuickSPI.inf)

- For enabling WoT (Wake on Touch) feature, install extension INF(QuickSPI-4.0.0.93/Desktop_Prod/x64/Release/ExtensionINFs/WoT_QuickSpiExtension.inf)

- Single IO Mode functionality can be verified till 31.25 MHz connection speed, due to panel limitation.

- Validated only in single IO mode due to BOM6 panel limitation.

# HW Setup with HIDSPI PCB

- Current driver package is validated with below HW setup.

BOM6 Panel:

Touch Cable 20-16 pin SPI (1.8V)

| Name | Platform | Display | Touch Vendor | Touch Protocol (Firmware) | Touch Cable | eDP Cable |
| --- | --- | --- | --- | --- | --- | --- |
| BOM6 | MTL | Sharp 12.5" 1W | Wacom G14T Fusion PCBA | HID-SPI over THC | 20-16 pin SPI (1.8V) | 60-pin |

# BIOS Menu Configuration

Below values can be configured by the user from BIOS menu, which are specific to Wacom (BOM6) panel.

BIOS settings for Single IO mode:

Intel Advanced Menu àPCH-IO Configurationà THC configurationà THC0à Enabled

    - Intel Advanced Menu → PCH-IO Configuration → THC Configuration → TH0 → Enabled

    - THC MODE → HID Over SPI

    - Input Report Body Address → [1000]

    - Input Report Header Address → [0]

    - Output Report Address → [4]

    - Write Opcode → [2]

    - Read Opcode → [B]

    - Intel Advanced Menu → PCH-IO Configuration → THC Configuration → HID Over SPI Limit Packet Size → [1]

    - Intel Advanced Menu → PCH-IO Configuration → Serial IO Configuration → Disable I2C 0 to I2C 5 and SPI 0 to SPI 2.

SECURITY CONFIGURATION:

THC CONFIGURATION: